Text file for COILBLD3.GIF, graphical instructions for

construction of high performance 1/4 wave Tesla resonators.

This series of diagrams shows details of the coil base and the

steps required to construct a high current ground terminal

without drilling into the coil form or permanently mounting

protruding hardware.

7) The first step in constructing a high current ground terminal

is to cut out a rectangle in the sealer at the base of the coil

form just below the winding. The coil winding should end just

above one corner of scribed area. The cut should be about two

inches long and one inch high. This section of sealer is then

peeled and scraped away to expose the bare plastic coil form. Use

a blade and deeply score the bared plastic in a "hatch" pattern.

8) Cut a strip of copper from some heavy copper sheet. The strip

of copper should fit inside of, and almost fill, the cleared and

scored area on the coil base. Round the corners of the copper

strip, then bend the strip gently until it conforms exactly to

the curvature of the coil form. The rounded and curved strip of

copper should fit neatly on the coil form in the prepared area.

9) This next step requires a piece of steel (with a flat surface)

and a small hammer. Gently pull the base wire free all the way to

the beginning of the winding. Trim this wire, leaving a 2-1/2

inch length. Scrap the base wire clean of all sealer and insul-

ation. Carefully wrap some heavy plastic or cardboard around the

coil to protect the winding from dings, but leave the base wire

extended and exposed. Position the coil so that the base wire may

be laid out flat on a small steel block or plate, then gently

peen the wire out with a small hammer. Copper wire is very

malleable, it will be possible to peen the round wire into a thin

flat strip. Trim the length again if required.

10) Clean and tin the inside curved surface of the copper strip.

Clean and tin the peened out base wire from the coil. Using a

clothes-pin, or other small clamp, position the flattened base

wire diagonally across the inside curve of the copper strip.

Solder the two together with a very hot iron. Avoid clumps or

blobs of excess solder. Clean the soldered area thoroughly with

solvent to remove all traces of rosin, oil, and dirt. Sand the

soldered area gently with abrasive paper to smooth out any rough

areas and high spots.

11) Using a cloth or lint-free wipe, clean the scored rectangle

on the coil form base with solvent, carefully removing all traces

of oil and dirt. Wipe down the copper ground terminal one last

time. Prepare some strips of waxed paper and an assortment of

rubber bands. Mix up a small batch of clear, two-part epoxy.

Smear the scored rectangle on the coil form with epoxy, then

smear the inside curve of the copper ground terminal. Fit the

ground terminal into place, cover with strips of wax paper, and

secure with a couple of rubber bands around the coil form. Allow

sufficient time to gel the epoxy firmly, but do not allow a

complete cure. Remove the rubber bands and the wax paper strips.

Gently scrape away excess epoxy from the surface of copper ground

terminal, then wipe the terminal surface clean with solvent.

If attention has been paid to detail and technique, the copper

ground plate should be nearly flush and firmly attached to the

bottom of the coil form just below the winding. Sometimes it may

be necessary to fill in a low spot or two with a second tiny

batch of epoxy.

12) This figure shows how effective this ground terminal is in

practice. It is quite easy to connect one inch ground strap

directly to the base of the coil with a couple of rubber bands

or a long strip of electrical tape. This connection is ideal for

removing the heavy RF current produced at the base of the coil.

This ground terminal is also nice in that it may be quickly

connected and disconnected, offering flexibility and ease of

setup. This system is also efficient if the base terminal is to

be used to feed RF current into the bottom of the coil, as in the

extra coil of the Tesla Magnifier.

Being flush mounted without drilling holes into the coil form,

this terminal preserves the electrical strength of a sealed coil

form. Without protuberances typical of other terminal types,

there is nothing to break off or damage. The same lack of pro-

tuberances makes it easy to store several coils close together

without worry of scratching or cutting the finishes. This

terminal is clean and professional looking.

COILBLD3.GIF      6/8/95           Graphics and text prepared by:

Richard T. Quick II

10028 Manchester Rd

Suite 253

Glendale  MO  63122  USA